OECOLOGIA BRASILlENSIS

Volume 111: Ecologia de Praias Arenosas do Litoral Brasileiro

ABSALAO, Ricardo S. & ESTEVES, André M. (editores), 1997, p.229-243.

Programa de Pós-Graduação em Ecologia -Instituto de Biologia - UFRJ, Rio de Janeiro· RJ.

ESTUDO DA MORFODINÂMICA DE PRAIAS COMPREENDIDAS ENTRE SAQUAREMA E MACAÉ, RJ

BENTES, A. M. L.; FERNANDEZ, G. B. & RIBEIRO, A. Y.

Resumo:

A variação da morfodinâmica de praias entre Saquarema e Macaé foi verificada, durante um ano, através de perfis transversais às praias e medição dos parâmetros oceanográficos. Os resultados mostraram que os perfis localizados entre Saquarema e Arraial do Cabo tiveram uma grande variação no ganho e perda de sedimentos, associados a maior variação da energia das ondas, e os perfis de Macaé e Barra de São João mostraram-se mais estáveis, devido a menor variação de energia das mesmas.

Palavra-chave: morfodinâmica de praias arenosas.

Abstract:

"Morphodynamcis of beaches between Saquarema and Macaé, RJ"

The morphodynamic response to oceanographic variables of beaches located between Saquarema and Macaé, Rio de Janeiro State, were monitored trough monthly topographic profiles during low water spring tide. The results showed a high topographic stability ofthe beaches located north of cabo Búzios (Macaé and Barra de São João) and high topographic variability ofthe profiles at Massambaba beach, located westward of cabo Frio.

Key -word: sandy beaches morphodynamics.